Table of Contents

1. PURPOSE	2

2. SCOPE	2

3. TERMS, DEFINITIONS, ABBREVIATIONS, AND ACRONYMS	2

4. PLANT MODEL IN MODELICA  	3

4.1 ARCHITECTURE	3

4.2 INTERFACE	4

4.3 MODEL FORMULATION	7

4.3.1 CORE ELECTRICAL MODEL (1)	8

4.3.2 MECHANICAL DYNAMICS (2)	9

4.3.3 POWER BALANCE (3)	10

4.3.4 STATOR INDUCTANCE VARIATION (4)	10

4.3.5 ROTATIONAL LOSSES POWER (6)	12

4.3.6 ROTATIONAL LOSSES TORQUE (7)	13

4.3.7 POLE TO PHASE VOLTAGE CALCULATION (9)	13

4.3.8 TEMPERATURE VARIATION (10)	14

5. INTEGRATION OUTSIDE MODELICA ENVIRONMENT	15

5.1 CORE INTERFACE	15

5.2 INTEGRATION IN THE ELECTRIC DRIVE LIBRARY	15

6. MODEL VARIANTS	17

# PURPOSE

This document describes the transient electro-mechanical model for Brushless Permanent Magnet Motor (BPM) and how to apply them in various simulation environments.

# SCOPE

This library includes global models of BPM motors. The application can be used in motor control algorithm development for various platforms (laundry, dish, refrigeration, dryer)  with the usage of  MIL, SIL, PIL, RT-PIL or HIL.

# TERMS, DEFINITIONS, ABBREVIATIONS, AND ACRONYMS

| TERM | DEFINITION |
| --- | --- |
| BPM | Brushless Permanent Magnet Motor |
| MIL | Model in the Loop |
| SIL | Software in the Loop |
| PIL | Processor in the Loop |
| RT-PIL | Real-Time Processor in the Loop |
| HIL | Hardware in the Loop |

# PLANT MODEL IN MODELICA

## ARCHITECTURE

At the top level, all model variants follow the same architecture pattern. The model consists          of a complete BPM motor electro-mechanical transient model (1) and a mechanical load model (2).

Top-Level variants due to mechanical load

## INTERFACE

The core interface remains the same for all top-level model variants.

| Inputs |  |  |  |
| --- | --- | --- | --- |
| Signal name | Description | Unit | Data type |
| i_va | Input voltage, phase a | V | Real |
| i_vb | Input voltage, phase b | V | Real |
| i_vc | Input voltage, phase c | V | Real |
| i_tau_load | Load torque | Nm | Real |
| i_temp_stat | Stator temperature | degC | Real |
| i_temp_rot | Rotor temperature | degC | Real |

| Outputs |  |  |  |
| --- | --- | --- | --- |
| Signal name | Description | Unit | Data type |
| o_ia | Output current, phase a | A | Real |
| o_ib | Output current, phase b | A | Real |
| o_ic | Output current, phase c | A | Real |
| o_id | Output current, axis d | A | Real |
| o_iq | Output current, axis q | A | Real |
| o_Ld | Stator inductance, axis d (debug output) | H | Real |
| o_Lq | Stator inductance, axis q (debug output) | H | Real |
| o_ea | Back-electromotive force, phase a | V | Real |
| o_eb | Back-electromotive force, phase b | V | Real |
| o_ec | Back-electromotive force, phase c | V | Real |
| o_omega_m | Rotor mechanical rotational speed | rad/s | Real |
| o_omega_e | Rotor electrical rotational speed | rad/s | Real |
| o_theta_m | Rotor mechanical position | rad | Real |
| o_theta_e | Rotor electrical position | rad | Real |
| o_tau_e | Electromagnetic torque | Nm | Real |
| o_tau_shaft | Torque delivered to the shaft | Nm | Real |
| o_tau_rot_loss | Output Rotational Losses torque | Nm | Real |
| o_pe_b | Motor total power - calculated using power balance equation | W | Real |
| o_pe_dq | Motor total power - calculated by v*i in dq coordinates | W | Real |
| o_pe_abc | Motor total power - calculated by v*i in abc coordinates | W | Real |
| o_p_shaft | Power delivered to the shaft | W | Real |
| o_p_rot_loss | Output Rotational Losses | W | Real |

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Name | Description | Unit | Data type |
| Rs0 | Initial stator winding resistance | Ohm | Real |
| Ld0 | Initial stator inductance in the d axis | H | Real |
| Lq0 | Initial stator inductance in the q axis | H | Real |
| phiPM1 | Initial PM flux amplitude of the fundamental harmonic | Vs | Real |
| NPP | Number of pole pairs | - | Real |
| Jrotor | Rotor's inertia | kg*m2 | Real |
| B | Friction coefficient | - | Real |
| Ts | Step size of the internal delay block (if used) | s | Real |
| TsRef | Reference temperature of the rotor (PM) | degC | Real |
| TrRef | Reference temperature of the stator | degC | Real |
| alpha_s | Stator resistance temperature coefficient | 1/degC | Real |
| d_magnet | Magnets temperature coefficient | 1/degC | Real |
| Rsa_aux | Additional stator winding resistance, phase a | Ohm | Real |
| Rsb_aux | Additional stator winding resistance, phase b | Ohm | Real |
| Rsc_aux | Additional stator winding resistance, phase c | Ohm | Real |
| Ls_Sat_enable | Stator inductance saturation enable | - | Real |
| I_pk0 ... I_pk7 | Stator inductance LUT input coefficients | A | Real |
| Ld_a0 ... Ld_a7 | Stator inductance LUT coefficients to estimate Ld = f(id) | H | Real |
| Lq_b0 ... Lq_b7 | Stator inductance LUT coefficients to estimate Lq = f(iq) | H | Real |
| Rot_Loss_enable | Rotational losses calculation enable | - | Real |
| Rot_Loss_omega_th | Rotational losses calculation speed threshold | rad/s | Real |
| Rot_Loss_Speed0 ... Rot_Loss_Speed7 | Rotational losses LUT input coefficients | rpm | Real |
| Rot_Loss_Power0 ... Rot_Loss_Power7 | Rotational losses LUT coefficient to estimate power_rot_loss = f(speed) | W | Real |

## MODEL FORMULATION

This document describes in detail only the complete BPM motor electro-mechanical model (1). Mechanical load models are not covered in this guideline - please refer to other available documentation. As shown on the diagram below, the Complete BPM model consists of several functional modules (components). Selected of them are described in detail in the following sections.

### CORE ELECTRICAL MODEL (1)

A first-principles model describing the electrical dynamics of a motor. Essential outcomes are the motor currents and the generated electromagnetic torque. Also, this module calculates the total motor power and Joule losses. Motor equations are expressed in the DQ reference frame, therefore the Clarke-Park transformation of selected signals is required to apply, in order to transform the model reference frame from ABC to DQ coordinates.

The inverse transformation can be easily derived from the above equations. It is mainly required for calculating motor phase currents. As for now, the zero (f0) component is not used in the model. Therefore, the implemented motor equations are based solely on the D and Q components:

- voltage equations

- permanent magnet flux equations (the PM flux is aligned to the D axis)

- stator flux equations

- currents dynamics

*Rs, ψm - can vary due to temperature,

*Ld, Lq - can vary due to the saturation effect.

- electromagnetic torque generation

- power calculations

  - total power in DQ reference frame

  - total power in ABC reference frame (more as a debug signal)

  - Joule power losses

### MECHANICAL DYNAMICS (2)

A first-principles model to calculate:

- mechanical rotor speed and position

- electrical rotor speed and position

- mechanical torque and power delivered to the shaft

### POWER BALANCE (3)

This module calculates the total power of the motor based on the power balance equation. According to theoretical considerations, the output power from this block should be equal to the total power calculated by the core model (1).

### STATOR INDUCTANCE VARIATION (4)

Stator inductance may vary during motor operation e.g. due to a saturation effect. An empirical approach is used to model this phenomenon. The approach relies on look up tables (LUTs) which calculate each inductance based on coefficients I_pk0 ... I_pk7, Ld_a0 ... Ld_a7, Lq_b0 ... Lq_b7 which are derived by identification procedures performed on the laboratory test bench. Inductance values in D and Q axis are calculated separately based on instantaneous  id and iq currents values respectively.

In general, inductances can be described as Ld = fid  , Lq = fiq where the function f is a linear interpolation of LUT points.

| Ipk [A]; Ld [H]. Ipk0; Ld_a1. Ipk1; Ld_a2. Ipk2; Ld_a3. Ipk3; Ld_a4. Ipk4; Ld_a5. Ipk5; Ld_a6. Ipk6; Ld_a7. Ipk7; Ld_a8. | Ipk [A]; Lq [H]. Ipk0; Lq_b1. Ipk1; Lq_b2. Ipk2; Lq_b3. Ipk3; Lq_b4. Ipk4; Lq_b5. Ipk5; Lq_b6. Ipk6; Lq_b7. Ipk7; Lq_b8. |
| --- | --- |

- Ld, Lq calculation example

### ROTATIONAL LOSSES POWER (6)

Rotational losses power calculation also relies on the lookup table approach. In this case, the coefficients are also derived from test procedures performed on the laboratory test bench.

In general, the rotational losses power can be described as pRL = f(n) where the function is the linear interpolation of rotational losses LUT coefficients.

| n [rpm] | p_RL [W] |
| --- | --- |
| n0 | p_RL0 |
| n1 | p_RL1 |
| n2 | p_RL2 |
| n3 | p_RL3 |
| n4 | p_RL4 |
| n5 | p_RL5 |
| n6 | p_RL6 |
| n7 | p_RL7 |

- rotational losses power calculation example

### ROTATIONAL LOSSES TORQUE (7)

Loss torque is considered as opposing torque in the mechanical dynamics of the motor as shown on the figure.

It is derived from:

### POLE TO PHASE VOLTAGE CALCULATION (9)

This module reconstructs the pole into the phase voltage passed to the core electrical BPM model. Also, it enables to simulate phase unbalance conditions by injecting additional resistances to each phase separately. Picture below shows a typical diagram of connecting a BPM motor with a voltage source inverter.

From a standpoint of the BPM input voltage, this electrical circuit can be described following equations:

where:

- Va ,Vb ,Vc - pole voltages

- Van ,Vbn ,Vcn - phase voltages

- Vn - potential in the neutral point

- ia ,ib ,ic - motor phase currents

### TEMPERATURE VARIATION (10)

This module calculates changes of the stator resistance and permanent magnet flux amplitude due to stator and rotor temperature variations. The outputs are multiplication coefficients (k_temp_Rs, k_temp_phiPM) passed to the core electrical model. Input temperatures are converted from degC to K.

# INTEGRATION OUTSIDE MODELICA ENVIRONMENT

In order to integrate the model outside the Modelica environment, a binary needs to be generated. Two basic binary forms are used in the case of the BPM model: FMU and S-Function.

## CORE INTERFACE

After binary generation the core interface remains the same as in the top-level Modelica model (section 6.2). All model inputs, outputs and parameters are exposed.

## INTEGRATION IN THE ELECTRIC DRIVE LIBRARY

Ultimately the model is embedded into the Electric Drive Library. Due to specific requirements, the model is integrated there by using customized methods.

In the Matlab/Simulink environment, the Simulink Top-Level models have an optimized interface, limited only to essential signals. Binaries are integrated at the Low-Level. In terms of models parameterization, customized Top-Level block masks have been designed as well.

Top-Level Model Mask

# MODEL VARIANTS

The BPM motor model has several core, top-level variants, which are mainly determined by:

- coupled mechanical load (Dyno, VA, HA, Compressor),

- binary form (FMU, S-Function),

- special use case (e.g. high speed range, PM flux harmonics).

| CONTRIBUTORS (Individuals who contributed to the content of this document) |  |  |  |
| --- | --- | --- | --- |
| NAME | USERID | NAME | USERID |
| Chinmay A. Kirtane | KIRTACA |  |  |
| Szymon A. Bednarz | BEDNASA |  |  |

| REVISION HISTORY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| REV | DESCRIPTION OF CHANGE | SECTION | BY | CN | DATE |
| A | Draft | All | KIRTACA | - | 01-Dec-2017 |
| B | Major update. Document reorganization. Alignment with the latest BPM transient model (v2.5.0). | All | BEDNASA | - | 06-Feb-2023 |